Programmer's Calculator		05/11/90

INTRODUCTION

Programmer's calculator (PC) is a utility for Atari Portfolio.  PC

calculates 32-bit integers in signed or unsigned mode.  PC converts

integers between binary, decimal, hex, and octal bases.  To start PC on

the Portfolio, type "pc.com" or just "pc". To quit PC, type 'q' or 'Q'.

DISPLAY

PC's numerical display is the first row below the the program's title.

PC displays up to thirty-two binary digits, eleven octal digits, ten

decimal digits, or eight hex digits   between the square brackets.

DIGIT KEY SYMBOLS

The left-hand side of the screen shows up the sixteen digit key symbols,

labeled 0-9, A-F.  PC illuminates only those digits used in the current base.

As the user strikes the corresponding key on the Portfolio keyboard, PC

flashes the corresponding digit key symbol.

OPERATOR KEY SYMBOLS

The right-hand side of the screen shows the twelve operator key symbols,

labeled +, -, *, /, %, ~, ^, ', |, &, >,and  <.  As the user strikes the

corresponding key on the Portfolio keyboard, PC flashes the corresponding

operator key symbol. The Operator keys behave as follows:

These  binary operators relate the value on the display  and  the

value placed in PC's internal accumulator by the previous operator or '='.

+ adds the accumulator and the display value.

- subtracts the display value from the accumulator.

* multiplies the display by the accumulator.

/ divides the accumulator by the display.

% remainders the accumulator by the display.

| logically or's the accumulator with the display.

& and's the accumulator the with display.

^ exclusive or's the accumulator with the display.

> shifts the accumulator right display places.

< shifts the accumulator left display places.

The '='  key or Enter key completes an operation.

There are two unary operators:

~ takes the 1's complement of display.

' takes the 2's complement of display.

EDITING THE DISPLAY VALUE

The backspace key deletes last digit entered. The Del key clears the display.

Hitting Del again clears the accumulator also.

HELP SCREEN

F1  displays help information.  Remember, F1 on the Portfolio  is

the "Fn" key and the "1" key typed simultaneously. Help scrolls

a quick reference on the key symbol meanings by typing the up or

down arrows as well as PgUp or PgDn. Remember, page up is the "Fn"

key and the "PgUp" key typed simultaneously. To quit  Help,

type ESC or Enter.

CHANGING BASES

The F2 key pops up a dialog box which allows the user to change  bases.

In the dialog box the user may move between entries using the up arrow

and down arrow keys.  The Enter key accepts the base on which the cursor

is shown. The user alternately may type  the first letter of the base to

immediately select that entry. The bottom right-hand corner of the screen

always indicates the current base as HEXadecimal, OCTal, BINary, or

DECimal.

TOGGLING BETWEEN SIGNED AND UNSIGNED OPERATION

The 'S' key toggles between signed and unsigned operation.

At the bottom right-hand corner of the screen the messages SIGNED and

UNSIGNED indicate signed and unsigned modes respectively.

NUMERIC LOCK

Although PC cannot automatically put the Portfolio into Numeric Lock,

the user can set the keyboard in numeric lock mode by simultaneously

pressing "Lock" and the "Atari" key. This allows the keyboard legends in

red to take force, simplifying keystrokes.

CAVEAT

PC only runs on the Atari Portfolio.  Due to some Atari-specific functions,

PC will lock up a standard personal computer.

Robert Heath,  CIS 71600,437